Dangerous Goods Transport

Standard Operating Procedures

Company Logo

Company Name

| Issue by Date | Verify by Date | Approved by Date |
| --- | --- | --- |

# Dangerous Goods Compliance Practices.

  - Policy on the Transport of Dangerous Goods

Company Name (Pty) is Freight Forwarder who responsible for handling dangerous goods shipment for transport by air and comply with current IATA Dangerous Goods Regulation (IATA DGR) latest edition and Thailand’s Dangerous Goods law and regulations.

The following regulations are directly applicable.

- Air Navigation Act B.E. 2497, as amended by the Air Navigation Act (No. 14) B.E. 2562, Chapter 1/3 Regulation of Transport of Dangerous Goods and Prohibited or Special Handling Items

- Notification of The Civil Aviation Authority of Thailand on Classification and List of Dangerous Goods Which May Endanger the Safety of The Aircraft or Persons on Board the Aircraft B.E. 2562

- Notification of The Civil Aviation Authority of Thailand on Classification and List of Dangerous Goods Which May Endanger the Safety of The Aircraft or Persons on Board the Aircraft no. 2 B.E. 2564

- RCAAT no.68 Application for and issuance of permission for operating Dangerous Goods and Prohibited or Special Handling Items 2024”

- RCAAT no. 73 “the operations regarding the Transportation of Dangerous Goods and Prohibited or Special Handling Items. 2024”

- Current edition of the ICAO —Technical Instructions for the Safe Transport of Dangerous Goods by Air (ICAO TI).

- Current edition of the IATA Dangerous Goods Manual

    - Approval of the Registration Certificate Dangerous Goods Operator

Dangerous goods can only be carried according to the International Civil Aviation Organization’s Technical Instructions for the Safe Transport of Dangerous Goods by Air (Technical Instructions), irrespective of whether the flight is wholly or partly within or wholly outside the territory of a State. Organizations that operate regarding the transport of dangerous goods by air must be registered certificate for dangerous goods operators with CAAT, except as identified in 1.2 General exception and 1.2.7 Items that may be carried by passengers and crews below. An additional approval or an exemption may be required to permit the transport of some dangerous goods (see 1.1.2 Forbidden Dangerous Goods below

Company Name (Pty) holds a CAAT approval to the registration certificate dangerous goods operator.

Choose 1 or 2 paragraphs below (Delete one paragraph)

- Company Name (Pty)’s policy accepts handling all classes of dangerous goods from shippers or other freight forwarders to transport by air

- Company Name (Pty)’s policy is not accepted to handling certain dangerous goods class 1, 4.2 and 7

    - Forbidden Dangerous Goods (ICAO TI 1; 1.1, 2.1 7; 4.2 IATA DGR 2.1, 1.4.2, RCAAT no. 73 Article 19 (2))

Any article or substance which, as presented for transport, is liable to explode, dangerously react, produce a flame or dangerous evolution of heat or dangerous emission of toxic, corrosive or flammable gases or vapours under conditions normally encountered in transport must not be carried on aircraft under any circumstance.

Certain dangerous goods known to meet the description above have been included in the Dangerous Goods List (Doc 9284 Table 3-1 or IATA DGR Tabel 4.2) with the word “Forbidden” shown in columns. It must be noted, however, that it would be impossible to list all dangerous goods which are forbidden for transport by air under any circumstance. Therefore, it is essential that appropriate care be exercised to ensure that no goods meeting the above description are offered for transport.

Dangerous Goods are forbidden, intended to include articles being returned to the manufacturer for safety reasons.

Certain dangerous goods, which are normally forbidden, may be specifically approved for air transport by the State of Origin and the State of the Operator (CAAT):

- To transport dangerous goods forbidden on passenger and/or cargo aircraft where Special Provision A1/A2 applies; or Special Provision A1/A2 applies; or

- for other purposes as specified in the ICAO Technical Instructions; provided that in such instances an overall level of safety in transport which is at least equivalent to the level of safety provided for in the Technical Instructions is achieved.

In instances of extreme urgency or when other forms of transport are inappropriate or full compliance with the prescribed requirements is contrary to public interest, the States concerned may grant an exemption from the provisions of the Technical Instructions provided that in such instances an overall level of safety in transport which is at least equivalent to the level of safety provided for in the Technical Instructions is achieved. For the purposes of exemptions, “States concerned” are the States of Origin, Operator, transit, overflight and destination. For the State of overflight, if none of the criteria for granting an exemption are relevant, an exemption may be granted based solely on whether it is believed that an equivalent level of safety in air transport has been achieved.

For example, since controls exist for quantities of some explosives which may be carried to or from specific airfields in Thailand, Air operators must seek advice from CAAT as to the suitability of the intended airfield of loading and unloading when Class 1 dangerous goods are being carried under an A2 Approval.

Dangerous goods carried in accordance with an exemption or approval must comply with the conditions on the exemption or approval, as well as those on the permanent approval unless these have been varied by the exemption or further approval.

## Scope

The provisions of this Dangerous Goods used by:

All personnel employed by Company Name (Pty) Ltd are allocated to prepare and classify dangerous goods in all branches of Company name (Pty) Ltd;

All contracted or third-party service providers contracted to Company name (Pty) Ltd and handling dangerous goods or related documentation; and

Any person seeking access to the dangerous goods designated areas of Company name (Pty) Ltd cargo facilities.

## Definition

Where terms used in this GM are defined in CAAT Regulations, they are used in accordance with meanings and usage given therein. A wide variety of terms are used throughout the world to describe facilities, procedures and concepts for the transport of dangerous goods operations. As far as possible the terms used in this document are those which have the widest international use.

“Appropriate national authority” means Any authority designated, or otherwise recognized, by a State to perform specific functions related to provisions contained in these Instructions. “Approval” means an authorization granted by the Director General of the Civil Aviation Authority of Thailand under article 13 for:

a) the transport of forbidden dangerous goods on passenger and/or cargo aircraft where the Technical Instructions state that such goods can only be carried with approval; or

b) any other purposes as provided in the Technical Instructions granted by the director general and shall be accordance with technical instruction supplement.

“Aviation Inspector” means Dangerous Goods Inspector-ASI-OPS-DGI that appointed by the Director General in accordance with the announcement of the Civil Aviation Authority of Thailand on Air Safety Inspector.

“Baggage” means the personal property of passengers or crew carried on an aircraft by agreement with the operator.

“Cargo” means any property carried on an aircraft other than mail and accompanied or mishandled baggage.

“Cargo aircraft” means Any aircraft, other than a passenger aircraft, which is carrying goods or property.

“Combination packaging” means a combination of packaging for transport purposes, consisting of one or more inner packaging secured in an outer packaging in accordance with the relevant provisions of ICAO TI Part 4.

“Competent authority” means anybody or authority designated or otherwise recognized as such for any purpose in connection with these Instructions.

Note. - This applies to radioactive material only

“Consignment” means one or more packages of dangerous goods accepted by an operator from one shipper at one time and at one address, receipted for in one lot and moving to one consignee at one destination address.

“Operations concerning Dangerous goods and Prohibited or Special Handling Items” means of receiving, storing, loading, unloading or transporting dangerous goods and prohibited or special handling items on board an aircraft

“Receiving” means the operation of receiving dangerous goods or prohibited or special handling items include inspecting and verifying the accuracy and completeness of documents, packaging, marking and labeling to shall be in accordance with Technical Instruction before accepting them for air transport.

“Storage” means the temporary storage of dangerous goods and prohibited or special handling items before loading them onto an aircraft, which shall be conducted in defined designated secure places and shall be in accordance with Technical Instruction.

“Loading” means the operation of loading dangerous goods and prohibited or special handling items from the storage area onto the aircraft for transport, by placing them appropriately in accordance with Technical Instruction for safety during the flight.

“Unloading” means the operation of unloading dangerous goods and prohibited or special handling items from the aircraft in accordance with Technical Instruction.

“Transport” means the operation of transporting dangerous goods and prohibited or special handling items on an aircraft, with safety measures during the flight and compliance with Technical Instruction and relevant regulations.

“Dangerous Goods and Prohibited or Special Handling Items Operating License” means a license for operations related to receiving, storage, loading, unloading or transporting dangerous goods and prohibited or special handing items on board an aircraft under Article15/27 or Article15/29

“Dangerous Goods” means articles or substances which are capable of posing a hazard to health, safety, property or the environment and which are shown in the list of dangerous goods in the Technical Instructions or Prescribed as dangerous goods by the General Director under Section 15/28 (1).

“Dangerous Goods Acceptance Checklist” means a document used to assist in the examination of the outer appearance of dangerous goods packaging and related documents to determine whether all relevant requirements have been met;

“Dangerous goods accident” means an occurrence associated with and related to the transport of dangerous goods by air which results in fatal or serious injury to a person or major property or environmental damage;

“Dangerous goods incident” means an occurrence, other than a dangerous goods accident, associated with and related to the transport of dangerous goods by air, not necessarily occurring on board an aircraft, which results in injury to a person, property or environmental damage, fire, breakage, spillage, leakage of fluid or radiation or other evidence that the integrity of the packaging has not been maintained. Any occurrence relating to the transport of dangerous goods which seriously jeopardizes the aircraft or its occupants is also deemed to be a dangerous goods incident.

“Dangerous Goods Serious Incident” means an incident involving circumstances indicating that there was a high probability caused by dangerous goods, which is stipulated by the Aircraft Accident Investigation Committee.

“Dangerous goods security” means measures or precautions to be taken by operators, shippers and others involved in the transport of dangerous goods aboard aircraft to minimize theft or misuse of dangerous goods that may endanger persons or property.

“Dangerous Goods Transport Document” means a document which is specified by the Technical Instructions and contains information about the dangerous goods it accompanies.

“Director General” means Director General of the Civil Aviation Authority of Thailand

“Exception” means a provision in ICAO Technical Instruction and Annex 18 or this Part which excludes a specific item of dangerous goods from the requirements normally applicable to that item.

“Exemption” means an authorization, other than an approval, granted by an appropriate national authority providing relief from the provisions of the Technical Instructions.

“Freight container” means see unit load device.

Note. - For the definition of freight container for radioactive material, see 2;7.1.3.

“Freight Forwarder” means a organization who offers the service of arranging the transport of cargo, mail, baggages or other items by air.

“Handling Agent” means an organization assigned to act on behalf of the air operator’s functions in receiving, storing, loading, unloading, or carrying out other any passengers or cargo related operations whole or partially.

“ID number” means a temporary identification number for entries in Table 3-1 – Dangerous Goods List – which have not been assigned a UN number.

“Incompatible” means describing dangerous goods which, if mixed, would be liable to cause a dangerous evolution of heat or gas or produce a corrosive substance.

“Inner packaging” means a packaging for which an outer packaging is required for transport.

“Inner receptacle” means a receptacle which requires an outer packaging in order to perform its containment function.

“Intermediate bulk container (IBC)” means any rigid or flexible portable packaging, other than those specified in Part 6;3 of these Instructions, as described in Chapter 6.5 of the UN Model Recommendations, that is designed for mechanical handling and is resistant to the stresses produced in handling and transport, as determined by tests.

Note. - IBCs are only authorized by these Instructions for UN 3077, Environmentally hazardous substance, solid, n.o.s. as provided in Packing Instruction 956.

“Intermediate packaging” means a packaging placed between inner packagings or articles and an outer packaging.

“International System of Units (SI)” means a rational and coherent system of units which provides the basis for the units of measurement used for air and ground operations as contained in Annex 5 to the Convention on International Civil Aviation.

“ISO” (standard) means an international standard published by the International Organization for Standardization (ISO – 1, ch. de la Voie-Creuse, CH-1211 Geneva 20, Switzerland).

“Jerrican” means a metal or plastic packaging of rectangular or polygonal cross-section

“Mail” means any letters or parcels which are delivered by Thailand Post or by authorized organizations, shall comply with the regulations of the Universal Postal Union;

“Net explosive mass (NEM)” means the total mass of the explosive substances, without the packagings, casings, etc. (net explosive quantity (NEQ), net explosive contents (NEC), or net explosive weight (NEW) are often used to convey the same meaning).

“Net quantity” means either:

- the mass or volume of the dangerous goods contained in a package excluding the mass or volume of any packaging material; or

- the mass of an unpackaged article of dangerous goods (such as UN 3166).

For the purposes of this definition, “dangerous goods” means the substance or article as described by the proper shipping name shown in Table 3-1, such as for “Fire extinguishers”, the net quantity is the mass of the fire extinguisher. For articles packed with equipment or contained in equipment, the net quantity is the net mass of the article, such as for lithium-ion batteries contained in equipment, the net quantity is the net mass of the lithium- ion batteries in the package.

“Operator” means a person, organization or enterprise engaged in or offering to engage in an aircraft operation.

“Outer packaging” means the outer protection of a composite or combination packaging together with any absorbent materials, cushioning and any other components necessary to contain and protect inner receptacles or inner packagings.

“Overpack” means an enclosure used by a single shipper to contain one or more packages and to form one handling unit for convenience of handling and stowage.

Note. - A unit load device is not included in this definition.

“Package” means the complete product of the packing operation, consisting of the packaging and its contents prepared for transport.

“Packaging” means one or more receptacles and any other components or materials necessary for the receptacles to perform their containment and other safety functions.

Note. - For radioactive material, see ICAO TI Part 2;7.1.3.

“Passenger aircraft” means an aircraft that carries any person other than a crew member, an operator’s employee in an official capacity, an authorized representative of an appropriate national authority or a person accompanying a consignment or other cargo.

“Pilot-in-command” means the pilot designated by the operator, or in the case of general aviation, the owner, as being in command and charged with the safe conduct of a flight.

“Prohibited or Special Handling Items” means Prohibited or Special Handling Items which may endanger the Safety of the aircraft or persons on board the aircraft, as designated by the Director General as Prohibited or Special Handling Items under Section 15/30 (1).

“Proper shipping name” means the name to be used to describe a particular article or substance in all shipping documents and notifications and, where appropriate, on packaging

“Public Aerodrome Operator” means the owner or operator of a licensed aerodrome providing services to the public, or the holder of a public airport operating certificate.

“Quality assurance” means a systematic programme of controls and inspections applied by any organization or body which is aimed at providing adequate confidence that the standard of safety prescribed by these Instructions is achieved in practice.

“Receptacle” means a containment vessel for receiving and holding substances or articles, including any means of closing.

“Relevant Persons in the transport of dangerous goods” means those engaged in the transport of dangerous goods as specified in Section 5.

“Relevant Persons in the Transport of prohibited or special handling items” means those engaged in the transport of prohibited or special handling items as specified in Section 5.

“Safety management system (SMS)” means A systematic approach to managing safety, including the necessary organizational structures, accountabilities, policies and procedures.

“Salvage packaging” means a special packaging into which damaged, defective, leaking or nonconforming dangerous goods packages, or dangerous goods that have spilled or leaked, are placed for purposes of transport for recovery or disposal.

“Serious injury” means an injury which is sustained by a person in an accident and which: requires hospitalization for more than 48 hours, commencing within seven days from the date the injury was received; or results in a fracture of any bone (except simple fractures of fingers, toes or nose); or involves lacerations which cause severe hemorrhage, nerve, muscle or tendon damage; or involves injury to any internal organ; or involves second- or third-degree burns, or any burns affecting more than 5 per cent of the body surface; or involves verified exposure to infectious substances or injurious radiation.

“Shipper” means the person who signs the contract of carriage with operator for the carriage of goods and whose signature appears on the air waybill or shipment record

“Shipper’s Declaration for Dangerous Goods” means a specific document detailing the class and division, list of dangerous goods, UN or ID number, packing group, quantity of dangerous goods, and the shipper's signature in according with Technical Instruction.

“Shipment” means the specific movement of a consignment from origin to destination

“Single packaging” means a packaging which does not require any inner packaging to perform its containment function during transport

“State of Destination” means the State in the territory of which the consignment is finally to be unloaded from an aircraft.

“State of Operator” means the state in which the operator’s principal place of business is located or, if there is no such place of business, the operator’s permanent residence.

“State of Origin” means the State in the territory of which the consignment is first to be loaded on an aircraft.

“State of Registry” means the State on whose register the aircraft is entered

“Radioactive Material” means any element or compound whose a component of its atomic structure is unstable and decays by releasing radiation, either existing in nature or created from a production or use nuclear material, from a production by a radiation generator, or from any other processes, not including radioactive material with a characteristics of nuclear material or radioactive material together with any contaminated solids, liquids, and gases within the packaging.

“UN or ID number” means the four-digit number assigned by the United Nations Committee of Experts on the Transport of Dangerous Goods to identify a substance or a particular group of substances. “ID number” means an identification number specified in the Technical Instructions for an item of dangerous goods which has not been assigned a UN number;

“Unit load device” means any type of freight container, aircraft container, aircraft pallet with a net, or aircraft pallet with a net over an igloo

“Technical instructions” means the Technical Instructions for the Safe Transport of Dangerous Goods by Air (Doc. 9284), approved and issued periodically in accordance with the procedure established by the ICAO Council.

## Abbreviations

ICAO 	– International Civil Aviation Organization

IATA 	– International Air Transport Association

IAEA 	– International Atomic Energy Agency

UN 	– United Nations

CAAT 	– The Civil Aviation Authority of Thailand

DGI	– Dangerous Goods Inspector

ICAO TI	– Technical Instructions for the Safe Transport of Dangerous Goods by Air (DOC 9284)

DGR 	– Dangerous Goods Regulations

## Commitment Statement

I declare that, To the best of my knowledge, the information contained in Company name (Pty)Ltd Procedures Manual for the transport of dangerous goods by air is true and accurate, the practices and procedures set out in this Manual will be implemented and maintained at all sites of Company name (Pty)Ltd

This Procedures Manual will be adjusted and adapted to comply with all future relevant changes to DG-CAAT Regulations, unless Company name (Pty)Ltd informs the Civil Aviation Authority of Thailand that it no longer wishes to convey dangerous goods by air.

Company name (Pty) Ltd will inform CAAT in writing of: minor changes / deviation to its Procedures Manual at least within 10 working days; and major planned changes, such as new acceptance, labeling, handling, storage and segregation procedures and or major building works which might affect compliance with relevant Civil Aviation legislation or change of site/address, at least 15 working days prior to the commencement/the planned change,

In order to ensure compliance with relevant Civil Aviation legislation, Company name (Pty) Ltd will cooperate fully with all inspections, as required, and provide access to all documents, as requested by authorized officers,

Company name (Pty)Ltd will inform CAAT of any serious security breaches and of any suspicious circumstances which may be relevant to aviation security in particular any attempt to conceal prohibited articles in consignments,

Company name (Pty)Ltd will ensure that all relevant staff receive appropriate training and are aware of their security responsibilities under the company’s Procedures Manual

Company name (Pty)Ltd will inform CAAT if it ceases trading; it no longer deals with conveyance of dangerous goods; or it can no longer meet the requirements of the relevant Civil Aviation legislation.

I shall accept full responsibility for this declaration.

Signature: _________________________________ Date: _____________________

Name surname, Managing Director or Authorize Person

# Organizational Structure

## Company Profile

Company name (Pty)Ltd (Head Quarters)

Full name and address: …

## Nature of business:

Please choose the type of business

- Shipper

- Freight Forwarder

- Handling Agent

- Regulated Postal Authority

## Physical Characteristics of Premises used for Storage of Dangerous Goods

Type(s) of Warehouse (if applicable):

- leased and not shared with other companies

Address (es) of dangerous goods processing / storage area(s):

| No. | Location | Site location | Address |
| --- | --- | --- | --- |
| 1 | Administration | Head Office |  |
| 2 | Location Site 2 |  |  |
| 3 | Location Site 3 |  |  |
| 4 | Location Site 4 |  |  |
| 5 | Location Site 5 |  |  |

## Dangerous Goods Coordinator

2.4.1	The holders of dangerous goods operating license and the holders of certificate for dangerous goods operator shall appoint a person or more for being dangerous goods coordinators.

2.4.2	The person whose name appears below has been appointed by the organization as the Management Representative responsible for the implementation, monitoring and supervision of the aviation safety controls prescribed by the CAAT Regulations.

2.4.3	Communication between CAAT and Company name (Pty) Ltd should be directed to this official. This official is senior enough in the organization to affect any directives which might be necessary in the interest of aviation security for dangerous goods. He/She has the necessary experience and qualifications for this responsibility.

Name:

Title: Dangerous Goods Coordinator

ID Number:

Phone:

E-mail:

2.4.4	Duties should cover the roles outlined as follows:

- Having knowledge and comprehension of dangerous goods regulations and receiving appropriate training in dangerous goods training programe relevant to their responsibilities.

- Providing consultation to the executive and personnel who have responsibility for the transport of dangerous goods.

- Coordination with CAAT aviation inspectors in carrying out regarding dangerous goods.

- Establishing operations procedures or operations manual including other related documents and updating such procedures or manual continually.

2.4.5	Additional responsibilities may include the following:

- The development, amendment and implementation of the overall dangerous goods operation manual or standard operation procedures.

- The development and promulgation of companywide Dangerous Goods safety standards and practices to ensure compliance with RCAAT no. 73.

- Ensuring that effective risk assessment and analysis are conducted regularly and that there is sufficient response capability in case of incidents and accidents

- Ensuring understanding of legislation and regulations relating to Dangerous Goods throughout the organization.

- Providing specialized advice to line management in all aviation safety functions regarding dangerous goods.

- Ensuring all staff dealing with dangerous goods and related documentation are trained and proper control of training records are established and maintained.

- Ensuring appointment of suitably qualified persons as dangerous goods officials in the respective branches of (Company name) (Pty) Ltd in a case where multiple branches exist.

- Ensuring all dangerous goods transport documents are retained according to the requirements

# Operational Procedures

Company name (Pty) Ltd will apply systems and methods to ensure that dangerous goods in their care are properly identified, classified, packed, accepted, labeled, handled, documented, stored and conveyed on an aircraft engaged in commercial air transport operations in a safe manner.

## Classification

3.1.1	Dangerous Goods are defined as those goods which meet the criteria of one or more of 9 Hazard Classes and where applicable, to one of 3 Packing Groups. The 9 Classes relate to the type of hazard. Some Dangerous Goods classes are further subdivided into divisions. Some articles or substances may have more than one hazard to indicate the various hazardous characteristics.

Dangerous goods classes which may endanger the safety of the Aircraft or persons on board the Aircraft to the following.

Class 1: Explosives

Division 1.1: Substances and articles which have a mass explosion hazard

Division 1.2: Substances and articles which have a projection hazard but not a mass explosion hazard

Division 1.3: Substances and articles which have a fire hazard and either a minor blast hazard or a minor projection hazard or both, but not a mass explosion hazard

Division 1.4: Substances and articles which present no significant hazard

Division 1.5: Very insensitive substances which have a mass explosion hazard

Division 1.6: Extremely insensitive articles which do not have a mass explosion hazard

Class 2: Gases

Division 2.1: Flammable gases

Division 2.2: Non-flammable, non-toxic gases

Division 2.3: Toxic gases

Class 3: Flammable liquid

Class 4: Flammable solids; substances liable to spontaneous combustion; substances which, on contact with water, emit flammable gases

Division 4.1: Flammable solids, self-reactive and related substances and solid desensitized explosives and polymerizing substances

Division 4.2: Substances liable to spontaneous combustion

Division 4.3: Substances which, in contact with water, emit flammable gases

Class 5: Oxidizing substances and organic peroxides

Division 5.1: Oxidizing substances

Division 5.2: Organic peroxides

Class 6: Toxic and infectious substances

Division 6.1: Toxic substances

Division 6.2: Infectious substances

Class 7: Radioactive material

Class 8: Corrosive substances

Class 9: Miscellaneous dangerous substances and articles, including environmentally hazardous substances which, during air transport, present a danger to the safety of the aircraft or persons on board the aircraft is not covered by Class (1) to (8) in according to notifications to prescribe by the Director General.

3.1.2 	Dangerous Goods are assigned to the relevant Packing Group according to the degree of Danger they present as follows:

Packing Group I 	- High Danger

Packing Group II	- Medium Danger

Packing Group III	- Low Danger

NOTE: It is the Shipper’s responsibility to identify and classify all Dangerous Goods intended for transport by road and air in compliance with regulations.

## Dangerous Goods Package

The Dangerous Goods where applicable are packed correctly as follows:

- The dangerous goods are packed in compliance with all applicable air transport requirements including:

  - inner packaging and the maximum quantity per package limits;

  - appropriate types of packaging according to the packing instructions;

  - other applicable requirements indicated in the packing instructions including:

- single packaging may be forbidden;

- only inner and outer packaging indicated in the packing instructions are permitted;

- inner packaging may need to be packed in intermediate packaging; and

- certain dangerous goods must be transported in packaging meeting a higher performance level.

- appropriate closure procedures for inner and outer packaging

- the compatibility requirements as specified in the applicable packing instructions

- the absorbent materials requirements in the packing instructions when applicable; and

- the pressure differential requirement

- The overpack must not contain packages enclosing different substances which might react dangerously with each other or packages of dangerous goods which require segregation according to IATA DGR Table 9.3.A. In addition packages containing UN 3090, lithium metal batteries prepared in accordance with Section IA or Section IB of PI 968 and UN 3480, lithium ion batteries prepared in accordance with Section IA or Section IB of PI 965 are not permitted in an overpack with packages containing dangerous goods classified in Class 1 other than Division 1.4S, Division 2.1, Class 3, Division 4.1 or Division 5.1

- Each package contained within an overpack must be properly packed, marked, labelled and be free of any indication of damage or leakage and in all respects be properly prepared as required in these Regulations. Packages must be secured within the overpack.

| Single Packaging | Packaging which does not require any inner packaging in order to perform their containment function during transportation |
| --- | --- |
| Inner Packaging | Packaging for which an outer packaging is required for transport |
| Outer Packaging | The outer protection of a composite or combination packaging together with components necessary to contain and protect inner receptacles or inner packaging |
| Combination Packaging | A combination of packaging for transport purposes, consisting of one or more inner packaging secured in an outer packaging |
| All Packed in One | Different classes of COMPATIBLE Dangerous Goods which are packed into one outer packaging |
| Overpack | An enclosure used by a single shipper to contain one or more packages of compatible Dangerous Goods and to form one handling unit for convenience of handling and storage |

## Dangerous Goods Marking

3.3.1	All necessary markings of Dangerous Goods must comply with the Regulations.

3.3.2	Proper Shipping Name – Dangerous Goods are assigned to UN Numbers and proper shipping names according to their hazard classification and their composition.

3.3.3	All labels must meet the quality and specification requirements of the Regulations and must be affixed in the correct location and in a secure manner. All markings must be visible, legible and so placed that they are not covered or obscured in any way.

3.3.4	Required Markings are:

- UN or ID Number and Proper Shipping Name

- Full Name and Address of Shipper and Consignee

- Hazard Label - must bear the Class and division number if applicable in the bottom corner label. Unless otherwise provided in the DGR, Hazard text indication the nature of the risk may be inserted in the lower half of the hazard label in addition to the hazard class, division or compatibility group

- Handling Label - give an additional information on how to handle, store or load Dangerous Goods

- The net quantity or gross weight as applicable for all classes (except ID8000, Class 7) (for non-identical consignments of more than one package)

- The net quantity for UN 1845 Carbon dioxide, solid (Dry Ice)

- For Infectious Substance (Division 6.2) the Name and Telephone Number of Responsible Person

- Package orientation labels (or pre-printed), “KEEP UPRIGHT”, “DO NOT DROP–HANDLE WITH CARE” for refrigerated liquefied gases

- UN3373: BIOLOGICAL SUBSTANCE CATEGORY B” and the diamond shaped mark from PI 650

- Chemical oxygen generators (when S.P. A144 applies) “Air Crew Protective Breathing Equipment (smoke hood) in accordance with Special Provision A144”

- Limited quantity mark for limited quantity packaging

- The word “SALVAGE” for salvage packaging to be used when transporting packages into which damaged, defective, leaking or non-conforming Dangerous Goods packages, or Dangerous Goods that have spilled or leaked and are placed for the purpose of transport for recovery or disposal

- Environmentally hazardous substance mark for environmentally hazardous substances

- Consolidated packages – multi packages from more than one person

- “Not Restricted” – if the package does not fall into the criteria of classification

- “Overpack” – if overpack used

Note 1: Should any of the above points be overlooked, the consignment will be rejected and returned to the Shipper immediately

Note 2: Example of the Hazard and Handling Labels and Marks as attached in appendix E

3.3.5 	Marks on dangerous goods packages prepared for transport by air aid in identifying the shipper, consignee and the description of the dangerous goods articles or substances within the package. The various marks applicable to the transport of dangerous goods by air and their descriptions are illustrated as follows:

| Marking |  |  |  |
| --- | --- | --- | --- |
| Limited Quantities Mark | Environmentally Hazardous Substance Mark | Battery Mark * Place for UN number(s) | Package Orientation |
| Excepted Quantity Package Mark | Surface Limited Quantities Mark | Biological substances Category B Mark | Genetically modified microorganisms (GMMOs) and organisms (GMOs) Genetically modified microorganisms Mark |

## Dangerous Goods Labels

Marks and labels are used to identify dangerous goods and communicate their hazards, handling and other information to operations personnel when such goods are transported by air as cargo consignments. Consequently, the presence of such marks and labels on the outer surface of packages intended for transport by air would suggest that its contents meet the criteria for classification as dangerous goods.

    - Hazard Labels

Some dangerous goods may have more than one hazardous characteristic and multiple dangerous goods articles or substances may also be packed in multiple inner packaging within an outer packaging. To communicate all the hazards of dangerous goods present within a package, dangerous goods transport regulations require applicable hazard labels to be affixed to the outer surface of packages containing such goods. Hazard labels are set in a diamond shape and recognizable by its symbol, class/division number and color representing the various hazards within the nine classes of dangerous goods. The hazard labels used in the transport of dangerous goods by air and their descriptions are illustrated as follows:

| Class 1 - Explosive |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Explosive (Divisions 1.1, 1.2, 1.3) |  | Explosive (Division 1.4) |  | Explosive (Division 1.5) |  | Explosive (Division 1.6) |  |
| * Place for Division and Compatibility Group, for example “1.1C”. |  |  |  |  |  |  |  |
| Class 2 - Gases |  |  |  |  |  |  |
| Flammable (Division 2.1) | Non-flammable, non-toxic (Division 2.2) |  | Toxic (Division 2.3) |  |  |  |
| Class 3 - Flammable Liquids |  |  |  |  |  |  |  |  |

| Class 4 - Flammable Liquids |  |  |  |
| --- | --- | --- | --- |
| Flammable Solids (Division 4.1) | Substances Liable to Spontaneous Combustion (Division 4.2) | Substances which in Contact with Water Emit Flammable Gases (Division 4.3) |  |
| Class 5 - Oxidizing Substances and Organic Peroxides |  |  |  |
| Oxidizing Substances (Division 5.1) | Organic Peroxides (Division 5.2) |  |  |
| Note: This label may also be printed with symbol (flame), text, numbers and borderline shown in black on red background. |  |  |  |

| Class 6 - Toxic and infectious substances |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Toxic Substances (Division 6.1) | Infectious Substances (Division 6.2) |  |  |  |  |  |  |
| Note: The lower part of Division 6.2 label should bear the inscription: INFECTIOUS SUBSTANCE In case of Damage or Leakage Immediately Notify Public Health Authority |  |  |  |  |  |  |  |
| Class 7 - Radioactive Material |  |  |  |  |  |  |  |  |
| Category I–White |  | Category II–Yellow |  | Category III–Yellow |  |  | Criticality Safety Index Label |  |
| Class 8 - Corrosives |  |  |  |  |  |  |  |  |
| Class 9 - Miscellaneous Dangerous Goods |  |  |  |  |  |  |  |  |
| Miscellaneous Dangerous Goods |  | Lithium Batteries or Sodium Ion Batteries |  |  |  |  |  |  |

    - Handling Labels

Handling labels affixed to dangerous goods packages are intended to communicate handling requirements or the nature of the dangerous goods. It should be noted that not all dangerous goods packages require handling labels. Unlike hazard labels, handling labels are set in a rectangle shape. The handling labels used in the transport of dangerous goods by air and their descriptions are illustrated as follows:

| Handling Labels |  |  |  |
| --- | --- | --- | --- |
| Magnetized Material | Cargo Aircraft Only | Cryogenic Liquids | Package Orientation |
| Keep Away From Heat | Radioactive Material - Excepted Package |  |  |

## Safety Data Sheet (SDS)

The SDS includes information such as the properties of each chemical; the physical, health, and environmental health hazards; protective measures; and safety precautions for handling, storing, and transporting the chemical.  The information contained in the SDS must be in English (although it may be in other languages as well). The SDS preparers may also include additional information in various sections.  Sections 1 through 8 contain general information about the chemical, identification, hazards, composition, safe handling practices, and emergency control measures (e.g., firefighting).  This information should be helpful to those that need to get the information quickly. Sections 9 through 11 and 16 contain other technical and scientific information, such as physical and chemical properties, stability and reactivity information, toxicological information, exposure control information, and other information including the date of preparation or last revision.  The SDS must also state that no applicable information was found when the preparer does not find relevant information for any required element.

The SDS must also contain Sections 12 through 15, to be consistent with the UN Globally Harmonized System of Classification and Labeling of Chemicals (GHS), but OSHA will not enforce the content of these sections because they concern matters handled by other agencies.

Section 14 – Transport Information purpose to cover transportation classification including UN number and proper shipping name, hazard class and division, packing group and ICAO/IATA/IMDG/DOT regulations.

## Dangerous Goods Transport Documentation

    - General

- The person who offers dangerous goods for transport by air must provide the air operator with the information applicable to the consignment as set out in this paragraph.

- Shipper’s Declaration for Dangerous Goods shall provide the operator with two copies, completed and signed. The signatory has been trained according to their responsibilities.

- Shall be provided Safety Data Sheet (SDS) or Material Safety Data Sheet (MSDS) or the authentication report issued by dangerous goods authentication

- Shall ensure that the dangerous goods described in the dangerous goods transport document, Safety Data Sheet or the report is identical to the contents of the consignment

    - Shipper’s Declaration for Dangerous Goods

- A Shipper’s Declaration for Dangerous Goods may be in any form, provided it contains all of the information required by these Instructions.

- If both dangerous and non-dangerous goods are listed in one document, the dangerous goods must be listed first or otherwise be emphasized.

- Continuation page. Shipper’s Declaration for Dangerous Goods may consist of more than one page, provided pages are consecutively numbered.

- The information on a Shipper’s Declaration for Dangerous Goods must be easy to identify, legible and durable

Note: information required on the shipper’s declaration for dangerous goods must contain the following IATA DGR 8.1

    - Air Waybill

Handling Information is required i.e. Air Waybill must have the following information: “Dangerous Goods as per associated shipper’s declaration or Dangerous Goods as per associated DGD”. Where dangerous goods are for the cargo aircraft only, the words “Cargo Aircraft Only” or CAO must be added. An airway bill containing both Dangerous and Non-Dangerous Goods should indicate the number of Dangerous Goods

If an article or substance could be suspected of being a dangerous goods, but does not meet the criteria for any of the hazard classes or divisions, it may be offered for transport as not restricted with the inclusion of a statement such as “not restricted”, “non-hazardous” or “non-dangerous” in the description of the article or substance on the air waybill to indicate that it has been checked. The statement “Not restricted, as per Special Provision Axx” must be included in the description of the article on the air waybill when required, to indicate that the special provision has been applied.

    - Additional Information

When shipments are shipped under exemption, a copy of the exemption must accompany the consignment

Note: Dangerous Goods Transport Document as above must be retained a minimum of 3 months after finishing the transportation. These documents shall be made available upon request by the CAAT

## Recognition of Undeclared / Hidden Dangerous Goods

    - General Description “Hidden Dangerous Goods”

Shippers and freight forwarders intending to offer general cargo by air transport may declare its goods under one or more general descriptions (e.g., chemicals, electrical equipment, ship spares etc.) instead of providing a breakdown or itemized list of items to accurately describe the nature of goods. The following table identifies the potential hidden dangerous goods that may be present within such general description of goods. The following is a list of general descriptions that are often used for items in cargo or in passengers’ baggage and the types of dangerous goods that may be included in any item bearing that description.

| General Description of Goods | Potential Hidden Dangerous Goods within such General Descriptions |
| --- | --- |
| Aircraft Spare Parts/ Aircraft Equipment/ Aircraft on Ground (AOG) Spares | May contain items meeting any of the criteria for dangerous goods. For example, explosives (flares or fire bottle cartridges), chemical oxygen generators, cylinders of compressed gas (such as oxygen, carbon dioxide, nitrogen or fire extinguishers), paint, adhesives, aerosols, life-saving appliances (life jackets, escape slides), fuel in equipment, first aid kits, wet or lithium batteries, matches, fire extinguishers, etc. |
| Automobile Parts/Supplies (car, car parts, motor, motorcycle) | May contain ferro-magnetic material which may not meet the definition for magnetized material, but which may be subject to special stowage requirements due to the possibility of affecting aircraft instruments (DGR 3.9.2.2). May also contain engines, including fuel cell engines, carburettors or fuel tanks which contain or have contained fuel, wet or lithium batteries, compressed gases in tyre inflation devices, fire extinguishers, shocks/struts with nitrogen, air bag inflators/air bag modules, flammable adhesives, paints, sealants and solvents, etc. |
| Battery-powered devices/Equipment | May contain wet or lithium batteries. |
| Breathing Apparatus | May indicate cylinders of compressed air or oxygen, chemical oxygen generators or refrigerated liquefied oxygen. |
| Camping/Expeditionary Equipment | May contain flammable gases (butane, propane, etc.), flammable liquids (kerosene, gasoline, etc.), flammable solids (hexamine, matches, etc.) or other dangerous goods. |
| Chemicals | Most chemicals are dangerous and only qualified dangerous goods acceptance staff can advise if it is acceptable as cargo. Dangerous chemicals must never be accepted as baggage. |
| COMAT (Company Materials) | Such as aircraft parts may contain dangerous goods as an integral part, e.g. chemical oxygen generators in a passenger service unit (PSU), compressed gas, flammable liquid, corrosive material, magnetized material, etc. |
| Consolidated Consignments (Groupages) | May contains any of the defined classes of dangerous goods. |
| Cryogenic (Liquid) | Indicates refrigerated liquefied gases such as nitrogen, neon, helium, argon, etc. Cryogenic liquids are dangerous because they might destroy human skin tissue on contact, and when spilled, they could cause suffocation in confined spaces. |
| Cylinders | Indicates compressed or liquefied gases. |
| Dental Apparatus | May contain flammable resins or solvents, compressed or liquefied gas, mercury and radioactive material. |
| Diagnostic Specimens | May contain infectious substances. |
| Diving Equipment | May include air cylinders (such as scuba tanks, vest bottles, etc.) which usually contain compressed air or a special gas mixture. Emptied cylinders (pressure gauge reads zero) are acceptable. Diving lamps may contain rechargeable lead acid batteries and high intensity diving lamps can generate extremely high heat when operated in air. In order to be carried safely, the bulb or battery must be disconnected. |
| Drilling or Mining Equipment | May contain explosives and/or other dangerous goods. |
| Dry Shipper (Vapour Shipper) | May contain free liquid nitrogen. Dry shippers are subject to the Regulations when they permit the release of any free liquid nitrogen irrespective of the orientation of the package. |
| Electrical Equipment/ Electronic Equipment | May contain magnetized materials or mercury in switch gear and electron tubes, wet batteries, lithium batteries or fuel cells or fuel cell cartridges that contain or have contained fuel. |
| Electrically Powered Apparatus (wheelchairs, mobility aids, lawn mowers, golf carts, etc.) | May contain wet batteries, lithium batteries or fuel cells or fuel cell cartridges that contain or have contained fuel. |
| Expeditionary Equipment | May contain explosives (flares), flammable liquids (gasoline), flammable gas (propane, camping gas) or other dangerous goods. |
| Film Crew or Media Equipment | May include explosive pyrotechnic devices, generators incorporating internal combustion engines, batteries, lithium batteries, fuel, heat producing items, etc. |
| Frozen Embryos | May contain refrigerated liquefied gas or Carbon dioxide, solid (dry ice). |
| Frozen fruit, vegetables, etc | May be packed in Carbon dioxide, solid (dry ice), which can adversely affect live animals and humans. |
| Fuels | May contain flammable liquids, flammable solids or flammable gases. |
| Fuel Control Unit | May contain flammable liquids. |
| Hot Air Balloon | May contain cylinders with flammable gas, fire extinguishers, batteries, etc. |
| Household Goods | May contain items meeting any of the criteria for dangerous goods including flammable liquids such as solvent based paint, adhesives, polishes, aerosols (for passengers, those not permitted under DGR 2.3), bleach, corrosive oven or drain cleaners, ammunition, matches, etc. |
| Instruments | May conceal barometers, manometers, mercury switches, rectifier tubes, thermometers, etc. containing mercury. |
| Laboratory/Testing Equipment | May contain items meeting any of the criteria for dangerous goods, particularly flammable liquids, flammable solids, oxidizers, organic peroxides, toxic or corrosive substances. |
| Machinery Parts | May contain adhesives, paints, sealants, solvents, wet and lithium batteries, mercury, cylinders of compressed or liquefied gas, etc. |
| Magnets and other items of similar material | May individually or cumulatively meet the definition of magnetized material. |
| Medical Supplies/ Equipment | May include items meeting any of the criteria for dangerous goods, particularly flammable liquids, flammable solids, oxidizers, organic peroxides, toxic or corrosive substances. |
| Metal Construction Material/Metal Piping/Metal Fencing | May contain ferro-magnetic material, which may be subject to special stowage requirements due to the possibility of affecting aircraft instruments DGR 3.9.2.2. |
| Passenger Baggage | May contain items meeting any of the criteria for dangerous goods. Examples include fireworks, flammable household liquids, corrosive oven or drain cleaners, flammable gas or liquid lighter refills or camping stove cylinders, matches, ammunition, bleach, aerosols (those not permitted under DGR 2.3), etc. |
| Pharmaceuticals | May contain items meeting any of the criteria for dangerous goods, particularly radioactive material, flammable liquids, flammable solids, oxidizers, organic peroxides, toxic or corrosive substances. |
| Photographic Supplies/Equipment | May contain items meeting any of the criteria for dangerous goods, particularly heat producing devices, flammable liquids, flammable solids, oxidizers, organic peroxides, toxic or corrosive substances. |
| Promotional Material | May contain items meeting any of the criteria for dangerous goods. |
| Racing Car or Motorcycle Team Equipment | May contain engines, including fuel cell engines, carburettors or fuel tanks which contain fuel or residual fuel, flammable aerosols, cylinders of compressed gases, nitromethane, other fuel additives, wet batteries, lithium batteries, etc. |
| Refrigerators | May contain liquefied gases or an ammonia solution. |
| Repair Kits | May contain flammable adhesives, solvent-based paints, organic peroxides, resins, etc. |
| Samples for Testing | May contain items meeting any of the criteria for dangerous goods, particularly infectious substances, flammable liquids, flammable solids, oxidizers, organic peroxides, toxic or corrosive substances. |
| Semen | May be packed with Carbon dioxide, solid (dry ice) or refrigerated liquefied gas. See also “Dry Shipper”. |
| Ships' Spares | May contain explosives (flares), cylinders of compressed gas (life rafts), paint, lithium batteries (emergency locator transmitters) etc. |
| Show, Motion Picture, Stage Special Effects Equipment | May contain flammable substances, explosives or other dangerous goods. |
| Sporting Goods/ Sports Team Equipment | May contain cylinders of compressed or liquefied gas (air, carbon dioxide, etc.), lithium batteries, propane torches, first aid kits, flammable adhesives, aerosols, etc. |
| Swimming Pool Chemicals | May contain oxidising or corrosive substances. |
| Switches in Electrical Equipment | May contain mercury. |
| Toolboxes | May contain explosives (power rivets), compressed gases or aerosols, flammable gases (butane cylinders or torches), flammable adhesives or paints, corrosive liquids, lithium batteries, etc. |
| Torches | Micro torches and utility lighters may contain flammable gas and be equipped with an electronic starter. Larger torches may consist of a torch head (often with a self-igniting switch) attached to a cylinder of flammable gas. |
| Unaccompanied Passenger Baggage/ Personal Effects | May contain items meeting any of the criteria for dangerous goods, such as fireworks, flammable household liquids, corrosive oven or drain cleaners, flammable gas or liquid lighter refills or camping stove cylinders, matches, bleach, aerosols, etc. (those not permitted under DGR 2.3). |
| Vaccines | May be packed with Carbon dioxide, solid dry ice. |

## Globally Harmonized System (GHS) Labelling

Some everyday household items bear consumer warning labels which may or may not indicate they are classified as dangerous goods in air transport. All over the world there are different laws on how to identify the hazardous properties of chemicals (called ‘classification’) and how information about these hazards is then passed to users (through consumer supply labels and safety data sheets for workers). This can be confusing because the same chemical can have different hazard descriptions in different countries.

For example, a chemical could be labelled for supply as ‘toxic’ in one country, but not in another. For this reason, the UN brought together experts from different countries to create the Globally Harmonized System of Classification and Labelling of Chemicals (GHS).

Products bearing the following GHS labels may indicate as dangerous goods:

Note - A product bearing the GHS corrosive label (depicted for right above) is NOT classified as dangerous goods if the signal word ‘Danger’ and hazard statement ‘causes serious eye damage’ applies. Products bearing the following GHS labels (and none of the above) are NOT classified as dangerous goods.

Table GHS Pictograms and their Criteria

| Pictogram |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Explosive | Gases Under Pressure | Flammable | Oxidizer | Toxic | Corrosive | Aquatic Toxicity |
| Appears on | Explosives |  | Gases Aerosols Liquids Solids | Oxidizing Gases Oxidizing Liquids Oxidizing Solids | Acute Toxicity Skin Oral Inhalation | Corrosive to Metal Skin Corrosion Serious Eye Damage | Acute Chronic |
|  | Self-Reactive Substances and Mixtures |  | Self-Reactive Substances and Mixtures Pyrophoric Liquids and Solids |  |  |  |  |
|  | Organic Peroxides |  | Self-Heating Substances and Mixtures |  |  |  |  |
|  |  |  | Substances and Mixtures, Which in Contact With Water, Emit Flammable Gases |  |  |  |  |
|  |  |  | Organic Peroxides |  |  |  |  |

Notes:

The GHS pictograms, as shown above, may indicate products which are dangerous goods for transport. There are, however, national and regional differences which may mean that packages bearing such pictograms are not classified as dangerous goods in transport.

When a substance is required to show corrosive symbol based solely on classification criteria of “serious eye damage” in the GHS classification, the substance does not meet the classification criteria of corrosive materials for transport.

| Pictogram |  |  |
| --- | --- | --- |
| Name | Harmful | Respiratory |
| Appear on | Harmful to Skin Oral Inhalation | Respiratory Sensitization Carcinogenicity Toxic to Reproduction |
|  | Skin Irritation Eye Irritation | specific Target Organ Systemic Toxicity Single Exposure |
|  | Respiratory Tract Irritation Narcotic Effects | Specific Target Organic Systemic Toxicity Repeated Exposure |
|  | Skin Sensitization | Aspiration Hazard Germ Cell Mutagenicity |

## Preparing Dangerous Goods Shipment (if applicable)

Before any package or overpack of dangerous goods is offered for air transport, the shipper must comply with the following specific responsibilities.

    - Classification

Company name (Pty) Ltd is responsible for identifying and classifying all dangerous goods intended for transport by air in compliance with IATA DGR. Specifically, before packing any dangerous goods for transport by air, the shipper must:

- identify, correctly and fully, all articles and substances that meet the criteria as dangerous goods within the consignment.

- classify each item of dangerous goods by determining which of the nine classes it falls and where relevant, determining any subsidiary hazards.

- where relevant, assign each item of dangerous goods to one of the three packing groups within the assigned class or division.

    - Identification

Dangerous goods must be assigned to one of the proper shipping names and UN number shown in the List of Dangerous Goods. Such substances may contain technical impurities (for example those deriving from the production process) or additives for stability or other purposes that do not affect their classification. However, a substance listed by name containing technical impurities or additives for stability or other purposes affecting its classification must be considered a mixture or solution. The proper shipping name is used to identify the dangerous article or substance on the outside of the package and on the “Shipper's Declaration for Dangerous Goods”.

    - Packing

Company name (Pty) Ltd is responsible for all aspects of the packing of dangerous goods in compliance with IATA DGR. When preparing each package of dangerous goods, Company name (Pty) Ltd must:

- comply with the set of packing requirements appropriate to the type of packaging to be used;

- use only the packagings permitted by the applicable packing instruction specified in Columns G, I and K of the List of Dangerous Goods;

- for all packagings, restrict the overall quantity per package to the limits specified in Columns H, J or L of the List of Dangerous Goods (as applicable) or to the design limit for the package whichever is more restrictive. In addition, for combination packaging, the quantity limit per inner packaging must not exceed the limits specified in the applicable packing instruction;

- assemble and secure all components of the packaging exactly in the manner intended;

- ensure that external surfaces of assembled package(s) are clean of contamination arising from the filling process itself or from contamination from the environment surrounding the filling/assembly area; and

- ensure that their responsibilities for packing are completely fulfilled when the package is presented to the operator for shipment.

    - Marking

For each package and overpack requiring one or more marks, Company name (Pty) Ltd must:

- check that any relevant mark on the package or overpack already on the package is in the correct location and meets the quality and specification requirements of IATA DGR;

- remove or obliterate any irrelevant mark already on the package or overpack;

- ensure that each outer or single packaging used for dangerous goods, for which specification packaging is required, bears the specification marks as specified in IATA DGR;

- apply any appropriate new mark in the correct location and ensure that it is of durable quality and correct specification;

- ensure that their responsibilities for marking are completely fulfilled when the package or overpack is presented to the operator for shipment.

    - Labeling

For each such package and overpack requiring labelling, Company name (Pty) Ltd must:

- remove or obliterate any irrelevant labelling already on the package or overpack;

- use only labels of durable quality and correct specification;

- inscribe on each label, in a durable manner, any required additional information;

- affix the appropriate label(s) in the correct location(s) and in a secure manner;

- ensure that the responsibilities for labelling are completely fulfilled when the package or overpack is presented to the operator for shipment.

    - Documentation

The dangerous goods must be providing information applicable to a consignment of dangerous goods to the operator as set out in IATA DGR. The information may be provided on a prescribed declaration form “Shipper's Declaration for Dangerous Goods” or, where an agreement exists with the operator, for each and every shipment containing dangerous goods so defined or classified in these Regulations unless it is stated that a Shipper's Declaration is not required. For each shipment containing dangerous goods Company name (Pty) Ltd must:

- use only the correct form in the correct manner.

- ensure that the information on the form is accurate, easy to identify, legible and durable;

- ensure that the form is properly signed when the shipment is presented to the operator for shipment; and

- ensure that the shipment has been prepared in accordance with IATA DGR.

## Receiving Dangerous Goods Shipment from Shipper/Freight Forwarder

3.10.1	A dangerous goods consignment which is prepared by shipper/freight forwarder will only be received by Company name (Pty)Ltd under the following conditions:

- Company name (Pty) Ltd only trained Staff and or third-party staff are trained on, which they are responsible prior to performing their duties.

- The goods are properly packaged with no evidence of leaking, tampering

- Proper documentation is available. That is, the Air waybill and a Shippers Declaration with the full name and address of Shipper and Consignee which will assists in terms of handling, loading and transporting of the dangerous goods as well as what exactly will be required in terms of emergency so as to aid response to any aircraft incident or accident

3.10.2	If the goods have been identified with the following:

- UN Number (United Nations Number)

- Class Division or Number

- Packing Group

3.10.3	The following is also critical:

- Number and Type of Packages

- Packing Instruction

- Any Exemption, approval and Special Provision (if applicable)

3.10.4 The properly dangerous goods package and completed dangerous goods transport document which be inspected by Company name (Pty) Ltd will be offered to air operator or ground handling to transport

## Transportation to Airport

- Only closed vehicles which protect the Dangerous Goods from sunlight will be used to transport Dangerous Goods to the airport

- Only appropriate Staff will collect the Dangerous Goods and transport them to Company name (Pty)Ltd warehouses

- Packages with Radioactive Material must be handled with care as far as possible from the person in order to maintain the principle of keeping exposure to radiation as low as possible

## Retention of Document

The following dangerous goods transport documents will be maintained for a period of not less than three months:

- Copies of shipper’s declarations

- Shipper's Letter of Instructions (if applicable)

- House Air waybills

- Master Air waybills

- Shipping documents (if applicable)

## Emergency Response Information (if applicable)

Company name (Pty)Ltd must ensure that for consignments requiring a Shipper's Declaration for Dangerous Goods, appropriate information is immediately available at all times for use in emergency response to accidents and incidents involving dangerous goods in air transport. The information must be available to the staff and can be provided by any document which provides appropriate information concerning dangerous goods.

24-hour emergency contact in Additional Handling Information of Shipper’s Declaration for Dangerous Goods must be prepared by shipper.

## Dangerous Goods Reporting Requirements

Dangerous goods occurrences meeting the criteria of The Civil Aviation Authority of Thailand requirement no. 22/2562 on "Reporting of Civil Aviation Occurrence" also meet the definition of a dangerous goods accident or incident (above), reportable in accordance with The Civil Aviation Authority of Thailand requirement no. 22/2562 on "Reporting of Civil Aviation Occurrence " Accordingly, the report must be reported to CAAT within 72 hours.

# Dangerous Goods Security

The security person responsible for Company name (Pty)Ltd engaged in the transport of high consequence dangerous goods adopt, implement and comply with a security plan that addresses the specified elements

Name:

Title:

ID Number:

Phone:

E-mail:

## Prohibited Articles

The following shall be considered as prohibited articles:

- Assembled explosive and incendiary devices that are not carried in accordance with the applicable safety rules.

- Live Intentionally Infected Animals or suspected to contain infectious substances.

## High Consequence Dangerous Goods (if applicable)

High consequence dangerous goods accepted into the facility will be stored in an area that is covered by CCTV. Where it is not possible to store such commodities in areas covered by CCTV, measures will be implemented to maintain constant surveillance over such commodities while under the care and control of the organization. Subject to the size and quantity of the consignment and where such consignment is required to remain overnight, the police authorities will be notified of the presence of such commodities.

Any loss or theft of high consequence dangerous goods will without delay be brought under the attention of the designated official. The incident shall be reported to the Civil Aviation Authority and followed up with a written report within 48 hours, if the incident occurs within the aerodrome area.

High Consequence Dangerous Goods are received from approved and known clients that declare the contents and ensure that goods are packaged properly. If the goods are not properly packaged, Company name (Pty)Ltd will send goods to Dangerous Goods Packaging Packers for repackaging.

## Premises, staff and access control and control of permits

4.3.1	Adequate physical barriers, such as fences, security gates and doors shall be installed as to

Physical barriers shall be of such nature that unauthorized access will be detected and prevented.

4.3.2	This will include the following:

- All shipping and receiving doors shall be closed and locked or guarded when not in use

- All shipping and receiving doors shall be equipped with intrusion detection devices. Where intrusion detection devices are not installed other means of protection against intrusion shall be used

- Access to Dangerous Goods areas shall be restricted to authorized persons with an operational need

- Access control shall be exercised at specific access control points, there being no other means of access into the premises other than through that point or points.

- Each access control point shall be staffed, effectively monitored or secured.

- The outer perimeter shall be clearly defined through physical barriers.

- The inner perimeter shall be clearly defined through physical barriers.

- Staff authorized to have unescorted access to controlled areas shall be issued with passes/permits.

- Visitors shall at all times be escorted if required to enter controlled areas within the cargo facility.

- A permit control process shall be implemented to approve and control the issuance, use and recovery of passes/permits.

## Unauthorized Access

4.4.1	Where an unauthorized person is found having access to Dangerous Goods areas, those goods will be inspected for any pilferage, leakage and or damage before being delivered by road to the airport or delivered from the warehouse to the aircraft.

4.4.2	In any such event of unauthorized access to Dangerous Goods areas, appropriate steps shall be taken without delay to prevent further breaches to the security system.

4.4.3	Where an unauthorized person is detected in the Dangerous Goods areas controlled area the following actions shall be taken:

- The person should be challenged. If it is regarded as not safe, the assistance of the security department and or police must be sought.

- The identity of the person should be established.

- The area where Dangerous Goods are kept that is intended for carriage by air shall be regarded as contaminated/unsecured.

- The area, including Dangerous Goods stored in the area, shall be subjected to security controls (which may include a thorough search/inspection) as to confirm that Dangerous Goods especially intended for carriage by air is secure.

## Protection of IT Systems

Computers and other electronic equipment are used for most operations and any sustained loss of power may result in negative effects on the business. The Uninterrupted Power Supply (UPS) systems are installed for most IT equipment to ensure business can continue as normally as possible in case of a power outage.

## Safeguarding, storage and maintenance of records

Documents are securely stored in lockable cabinets and or lockable offices and access to such documentation is controlled and restricted to authorized personnel only.

## Personnel Records

Personnel records are securely stored in lockable cabinets and or lockable offices and access to such documentation is controlled and restricted to authorized personnel only. These documents shall be made available upon request by the CAAT

- Dangerous Goods Training

## General

All organization shall establish and maintain initial and recurrent dangerous goods training program to be submitted to the Director General for approval under RCAAT no. 73 Chapter 5.

Personnel must be trained and assessed commensurate with the functions for which they are responsible prior to performing any of these functions.

A training program must include elements such as methodology, assessment, initial and recurrent training, instructor qualifications and competencies, training records and evaluation of the effectiveness of training

## Approval of Dangerous Goods Training Programmes

Choose 1 or 2 paragraphs below

- Company name use the training service provider who had been approved dangerous goods training program and registered by CAAT for providing dangerous goods training to Company name ‘s personnel.

- Company name approval for dangerous goods training programmes in the carriage of dangerous goods by air in accordance with CAAT requirement. This training is identified and described in the following text. Any substantive changes to this training must be submitted to the CAAT for the training approval to remain valid.

## General Requirements Applicable to Dangerous Goods Training Programmes

To ensure that everyone involved is aware of their responsibilities in the transport of dangerous goods, no matter whether such goods are carried as cargo or are in the possession of passengers, The employer of personnel that perform functions aimed at ensuring that dangerous goods are transported in accordance, ICAO Doc 9284 must establish and maintain a dangerous goods training programme.

A training programme includes elements such as methodology initial and recurrent training and assessment, instructor qualifications and competencies, training and assessment records and evaluation of its effectiveness. Operators need to determine the purpose and objective of the competency-based training programme based on the functions for which their personnel are responsible. The operator should ensure that training is designed and developed to establish clear links among the competencies to be achieved, the learning objectives, assessment methods, and course materials.

An approach to ensuring personnel is competent to perform any function for which they are responsible is provided in CAAT Guidance Material for Competency-based Approach to Dangerous Goods Training and Assessment (CAAT-GM-OPS-DGCBT).

## Objective of Dangerous Goods Training

Company names must ensure that personnel are competent to perform any function for which they are responsible prior to performing any of these functions. This must be achieved through training and assessment commensurate with the functions for which they are responsible. Such training must include:

- general familiarization training - Personnel must be trained to be familiar with the general provisions;

- function-specific training - Personnel must be trained to perform competently any function for which they are responsible; and

- safety training - Personnel must be trained on how to recognize the hazards presented by dangerous goods, on the safe handling of dangerous goods, and on emergency response procedures.

## Recurrent Training

Personnel must receive recurrent training and assessment within 24 months of previous training and assessment to ensure that competency has been maintained. However, if recurrent training and assessment is completed within the final three months of validity of the previous training and assessment, the period of validity extends from the month on which the recurrent training and assessment was completed until 24 months from the expiry month of that previous training and assessment.

As with other aviation qualifications, an offence against the regulations will be committed if staff continue to work after their training qualification has expired.

## Dangerous Goods Training Record

Company names must maintain a record of training and assessment for personnel.

The record of training and assessment must include:

- the individual’s name and surname:

- the month of completion of the most recent training and assessment;

- a description, copy or reference to training and assessment materials used to meet the training and assessment requirements;

- the name and address of the organization providing the training and assessment; and

- evidence which shows that the personnel have been assessed as competent.

Training and assessment records must be retained by the employer for a minimum period of 36 months from the most recent training and assessment completion month and must be made available upon request to personnel or the appropriate national authority.

## Instructor Qualifications

Instructors of initial and recurrent dangerous goods training programmes must have

- Having knowledge and comprehension of the Technical Instructions or IATA Dangerous Goods Regulations, including Thai regulations related the transport of dangerous goods by air,

- Have the capability to develop lesson plans, training materials, exercises, and training evaluation forms, including having completed training in instructional techniques.

- Instructors shall demonstrate or be assessed as competent in instruction and the function(s) that they will instruct prior to delivering such a dangerous goods training program.

- Having experience in the transport of dangerous goods or air cargo operations or other experiences related to flight operations for at least five (5) years