| No | Subject | Reference | S | U | N/A | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | General - Record Retention |  |  |  |  |  |
| 1.1 | Record Retention: Are operational records and reports used for flight preparation and execution stored for at least 3 months, including: (1) Operational Flight Plan (OFP) & Flight Release (2) Route-specific NOTAMs & AIS briefings (if edited by the operator) (3) Mass & Balance documentation (4) Notification of Special Loads/Dangerous Goods (NOTOC) (5) Journey Log or equivalent 6) Flight/Occurrence reports | ORO.MLR.115(b) |  |  |  |  |
| 1.2 | Ground Retention: Is the information relevant to the flight (OFP, Tech Log copies, NOTAMs, M&B, and special load notifications) preserved on the ground for at least the duration of the flight? | CAT.GEN.MPA.185 |  |  |  |  |
| 1.3 | Are data entries accurate and consistent across all documents? Note: For example, are the fuel records in the technical log/journey log identical to the fuel data in the operational flight plan? | ORO.MLR.110 CAT.GEN.MPA.185 |  |  |  |  |
| 2 | Aircraft Journey Log / Technical Log |  |  |  |  |  |
| 2.1 | Does the aircraft journey log, or equivalent record, contain all required information and is it properly completed, recorded, and retained for each flight or series of flights, as applicable? | ORO.MLR.110 AMC1 ORO.MLR.110 |  |  |  |  |
| 2.2 | Has the journey log for particulars of the aircraft, its crew and each journey been properly recorded and retained for each flight, or series of flights? | ORO.MLR.110 |  |  |  |  |
| 2.3 | Are flight times recorded flight times accurate for the routes and realistic? Consistent with FTL requirements? | ORO.MLR.100 AMC3 ORO.MLR.100 ORO.FTL |  |  |  |  |
| 2.4 | Aircraft Technical Log: - Fuel & Oil Consistency Are fuel/oil replenishment entries complete and consistent Note: Compare fuel figures recorded with OFP. | ORO.MLR.110, CAT.OP.MPA.181, CAT.OP.MPA.185(a)(4) |  |  |  |  |
| 2.5 | Aircraft Technical Log: - Defect reporting Are defects correctly recorded / deferred in accordance with MEL? | CAT.GEN.MPA.100(b)(14), CAT.GEN.MPA.180(a)(10) |  |  |  |  |
| 2.6 | Aircraft Technical Log: - Maintenance Record: Are routine maintenance statements correct? De-icing correctly recorded? | ORO.MLR.100, AMC3 ORO.MLR.100, CAT.GEN.MPA.180(a)(10) |  |  |  |  |
| 2.7 | Aircraft Technical Log: - Maintenance Release: Is there a valid release to service signed by authorized staff prior to the specific flight? | CAT.GEN.MPA.180(a)(10), Continuing Airworthiness Req. |  |  |  |  |
| 3 | Operational Flight Plan (OFP) |  |  |  |  |  |
| 3.1 | Does the OFP contain all required items? | AMC1 CAT.OP.MPA.175(a) |  |  |  |  |
| 3.2 | Is OFP prepared for correct aircraft? - Registration - Configuration - Limitations/MEL | CAT.OP.MPA.175 |  |  |  |  |
| 3.3 | Is the OFP signed by the PIC and (if applicable) the Flight Dispatcher to certify planning for weather, fuel, and performance? Are all required fields completed by crews? | CAT.OP.MPA.175 ORO.FOO/FD.010 |  |  |  |  |
| 3.4 | Fuel Planning: Are takeoff fuel, trip fuel, and final reserve fuel clearly documented? All fuel elements correct (e.g. alternate, contingency)? Are fuel orders appropriate for sectors and weather conditions - departure and arrival? | CAT.OP.MPA.180 CAT.OP.MPA.181 CAT.OP.MPA.182 AMC1 CAT.OP.MPA.175 (a) (b) |  |  |  |  |
| 3.5 | Fuel Monitoring: - Were periodic en-route fuel checks recorded to validate planning assumptions against actual consumption? - Is the final usable fuel on board upon arrival recorded to ensure final reserves were protected? | CAT.OP.MPA.185 |  |  |  |  |
| 3.6 | ATC flight plan correct for route? Note: ATC flight plan is normally part of electronic OFP. For a manual OFP, the ATC flight plan is normally a separate document | CAT.OP.MPA.175 AMC1 CAT.OP.MPA.175(a) |  |  |  |  |
| 3.7 | Does the OFP/ATS plan reflect navigation specs (e.g., RNAV/RNP) consistent with aircraft capability and route requirements? | CAT.OP.MPA.175(b)(7a) AMC1 CAT.OP.MPA.175 CAT.OP.MPA.126 |  |  |  |  |
| 3.8 | Destination, en-route, departure alternates correct for the weather conditions & crew qualifications? Note: may need to refer to TAFs and/or crew training records. | CAT.OP.MPA.182 CAT.OP.MPA.175(b)(7) ORO.FC.105 |  |  |  |  |
| 4 | Performance / Mass & Balance |  |  |  |  |  |
| 4.1 | Company calculation sheets or EFB output Aircraft operated within AFM performance limits? Any MEL/CDL limitations? | CAT.POL.MAB.100 CAT.POL.MAB.105 CAT.POL.A/H |  |  |  |  |
| 4.2 | Manual/electronic loadsheets or EFB output Aircraft operated within AFM mass & balance limits? Any MEL/CDL limitations? Calculations based upon correct - aircraft registration? - configuration? - crew? - catering? | CAT.POL.MAB.100 CAT.POL.MAB.105 |  |  |  |  |
| 4.3 | Are there signatures from both the loading supervisor (confirming distribution) and the commander (acceptance)? | CAT.POL.MAB.105(c) |  |  |  |  |
| 4.4 | Are Last Minute Changes (LMCs) clearly noted, or is a new document generated if limits were exceeded? | CAT.POL.MAB.105(d) |  |  |  |  |
| 5 | Weather data & NOTAMs |  |  |  |  |  |
| 5.1 | METAR, TAF, wind & significant weather charts, NOTAM, SNOTAM, ASHTAM Required documents retained? Note: When PBN relies on GNSS systems for which RAIM is required for integrity, its availability should also be verified. | ORO.MLR.115 (b) (2) CAT.OP.MPA.175 (b) (4) AMC1 CAT.OP.MPA.175 CAT.GEN.MPA.180 AMC1 CAT.GEN.MPA.180 (a)(18) |  |  |  |  |

S = Satisfied

U= Unsatisfied

N/A= Not Applicable

Comment= Description the detail of compliance or Non Compliance or other information

Inspector Name:………………………………………………………………….………

(………………………………….………..……………………….….)

Position:…………………………………………….…………………………………………

Completed on:…………………………………………………………………………….