The CAMO Product Audit is made of 3 phases: The aircraft records review, the aircraft management system, and the aircraft physical inspection. 
The comment will be used to record sample subjects and collected evidence.

| No | Subject | Reference | S | U | N/A | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRODUCT INFORMATION |  |  |  |  |  |
|  | Aircraft Type / Model / Series (Note: Use one checklist for one product line.) | CAMO.A.135 CAMO.A.315 |  |  |  |  |
|  | Aircraft Registration Mark and MSN |  |  |  |  |  |
|  | A/C TCDS No. and Rev. |  |  |  |  |  |
|  | Engines Type |  |  |  |  |  |
|  | ESN (#1, #2, #3, #4, …) |  |  |  |  |  |
|  | Engines TCDS No. and Rev. |  |  |  |  |  |
|  | Propellers Type |  |  |  |  |  |
|  | PSN (#1, #2, #3, #4, …) |  |  |  |  |  |
|  | Propellers TCDS No. and Rev. |  |  |  |  |  |
|  | Aircraft Operator name |  |  |  |  |  |
|  | Aircraft Operator Certificate’s number and validity (as applicable) |  |  |  |  |  |
|  | Aircraft Operation Specifications (as applicable): |  |  |  |  |  |
|  | No. and Rev. |  |  |  |  |  |
|  | The aircraft type is listed in the AOC’s Operations Specifications. |  |  |  |  |  |
|  | The aircraft registration is listed in the Attachment to Operations Specifications. |  |  |  |  |  |
|  | Granted specific approvals |  |  |  |  |  |
|  | Other active authorizations (Special Flight Permit, Operating license, etc.) |  |  |  |  |  |
|  | C of R Issue Date |  |  |  |  |  |
|  | C of A validity |  |  |  |  |  |
|  | For leased aircraft, lease agreement validity: | M.A.201 CAMO.A.135 ORO.AOC.110 ORO.AOC.115 |  |  |  |  |
|  | Ownership |  |  |  |  |  |
|  | Period and duration |  |  |  |  |  |
|  | Continuing airworthiness responsibility |  |  |  |  |  |
|  | AIRCRAFT RECORDS REVIEW |  |  |  |  |  |
|  | The aircraft complies with the latest approved type design: | CAMO.A.315 |  |  |  |  |
|  | The aircraft, engines and propellers comply with the latest TCDS. |  |  |  |  |  |
|  | The aircraft LOPA/ interior accommodation complies with approved configuration. |  |  |  |  |  |
|  | Check the aircraft approved mass (MTOM, MLM, etc.) |  |  |  |  |  |
|  | The aircraft, engines and propellers’ flight hours and cycles are properly recorded: | CAMO.A.315 CAMO.A.220(a) |  |  |  |  |
|  | Records reflect the status of the aircraft and hours/cycles are up to date. |  |  |  |  |  |
|  | Review hours/cycles up-date on ACARS, ATL (aircraft, engine, propeller, APU logbook). |  |  |  |  |  |
|  | All maintenance due on the aircraft is accomplished in accordance with the approved maintenance program: | CAMO.A.315 ORO.AOC.125 TCAR OPS Part SEA Subpart MCF SPO.SPEC.MCF NCO.SPEC.MCF |  |  |  |  |
|  | Review the approval and update of the maintenance program. |  |  |  |  |  |
|  | Sample that the aircraft maintenance records are in line with the content of the approved maintenance program. |  |  |  |  |  |
|  | Check that the maintenance program is customised to the aircraft condition. |  |  |  |  |  |
|  | Sample intervals between letter checks (A, C, D checks). |  |  |  |  |  |
|  | Sample out-of-phases tasks accomplishment and intervals. |  |  |  |  |  |
|  | Sample intervals between any extension and permitted variations. |  |  |  |  |  |
|  | Sample accomplishment of maintenance tasks: |  |  |  |  |  |
|  | Line maintenance task(s) |  |  |  |  |  |
|  | Base maintenance task(s) |  |  |  |  |  |
|  | Component replacement(s) |  |  |  |  |  |
|  | Specialised maintenance task(s) e.g. NDT, painting, etc. |  |  |  |  |  |
|  | Sample accomplishment of any maintenance deferrals. |  |  |  |  |  |
|  | Sample life limited and time limited components status and change records. |  |  |  |  |  |
|  | Sample airworthiness limitations and certification maintenance requirements. |  |  |  |  |  |
|  | Sample accomplishment of any Maintenance Check Flight (MCF) |  |  |  |  |  |
|  | All applicable ADs have been applied and recorded for airframe, engines, propeller, APU and appliances: | CAMO.A.315 |  |  |  |  |
|  | Review AD status vs State of Design issued AD’s and CAAT’s. |  |  |  |  |  |
|  | For AD’s including repetitive inspections, sample the inclusion of these inspection requirements in the management system and the interval between inspections already accomplished. |  |  |  |  |  |
|  | Sample AD evaluation and accomplishment records. |  |  |  |  |  |
|  | For AMOC’s to AD’s, verify they are CAAT approved/accepted. |  |  |  |  |  |
|  | All known defects have been properly rectified or carried forward: | CAMO.A.315 ORO.MLR.105 SEA.MLR.105 NCO.GEN.155 |  |  |  |  |
|  | Review the current ‘Deferred/carried forward Defect’ list for open defects affecting aircraft operation. |  |  |  |  |  |
|  | Sample that defects were deferred in accordance with CAAT approved maintenance data (MEL, AMM, CDL, DDG, etc.). |  |  |  |  |  |
|  | Sample that defects are rectified within the delay/limits granted by the approved maintenance data. |  |  |  |  |  |
|  | Control of any repetitive defects. |  |  |  |  |  |
|  | All modifications applied are recorded and CAAT approved: | CAMO.A.315 |  |  |  |  |
|  | Review SB’s and modifications status. |  |  |  |  |  |
|  | Sample modifications files to ensure they are CAAT approved/accepted (STC, DOA, Part-21 approved modifications, etc.). |  |  |  |  |  |
|  | Sample SB’s and modifications incorporation records. |  |  |  |  |  |
|  | Sample of maintenance tasks introduced by modification including its documentation. |  |  |  |  |  |
|  | All supplemental documents (Supplement AFM, MEL, AMM, IPC, ELA, ICA, etc.) are available and up to date. |  |  |  |  |  |
|  | All repairs applied are recorded and CAAT approved/accepted. |  |  |  |  |  |
|  | Review repair status (dent and buckle chart or damage marking). |  |  |  |  |  |
|  | Sample repairs files to ensure they are CAAT approved/accepted (EO, Part 21 approved repairs, etc.). |  |  |  |  |  |
|  | Sample repairs incorporation records. |  |  |  |  |  |
|  | Sample repetitive maintenance task from repairs or allowable damages. |  |  |  |  |  |
|  | Sample in-service limits from repairs or allowable damages. |  |  |  |  |  |
|  | The current mass and balance statement reflects the aircraft configuration. | CAMO.A.315 CAT.POL.MAB SEA.CSO.MAB SPO.POL.105 NCC.POL.105 NCO.POL.105 |  |  |  |  |
|  | Control that mass and balance report reflects the current aircraft configuration and is valid. |  |  |  |  |  |
|  | Periodical aircraft weighing is accomplished in accordance with the operator’s policy/approved maintenance program. |  |  |  |  |  |
|  | Control that aircraft weighing is done after modifications or repairs, as required. |  |  |  |  |  |
|  | The mass and balance report are fully represented in aircraft prepared for service weights and indices used in the loading calculations made prior to flight dispatch. |  |  |  |  |  |
|  | The maintenance is always released to service by CAAT approved maintenance organisations and recorded in aircraft continuing airworthiness record system and applicable aircraft technical log system. | CAMO.A.220 CAMO.A.315 |  |  |  |  |
|  | Sample aircraft technical log system, aircraft logbook, engine logbooks, propeller logbooks, APU logbook entries and components release to services. |  |  |  |  |  |
|  | Sample the use of release to service documentation (statement and required minimum information). |  |  |  |  |  |
|  | The status of aircraft operational-derived equipment meets operational requirements: | CAMO.A.315 CAT.GEN.MPA.195 CAT.IDE.A/H SEA.CSO.IDE ORO.AOC.105 TCAR OPS Part-SPA SPO.GEN.145 SPO.IDE.A/H NCC.GEN.145 NCC.IDE.A/H NCO.IDE.A/H TCAR OPS Part-SPO Subpart SPEC TCAR OPS Part-NCO Subpart SPEC |  |  |  |  |
|  | Installation and tasks for instrument and equipment for operations, communication, navigation, and surveillance, including but not limited to: - Safety equipment (depends on the aircraft’s type of operation) - Flight recorders (including its documentation and recording inspections) - ELT coding and registration - Mode-S address field test |  |  |  |  |  |
|  | Installation and maintenance for instrument/equipment/system for granted specific approvals or specialised operations or other authorizations. |  |  |  |  |  |
|  | Control of electronic aeronautical database management, as applicable: - Navigation database Terrain database Other software application |  |  |  |  |  |
|  | Control of lists containing information of the emergency and survival equipment carried on board. |  |  |  |  |  |
|  | Accomplishment of pre-flight inspections | CAMO.A.315 |  |  |  |  |
|  | As applicable, reliability report of this aircraft type including: | CAMO.A.315 M.A.302(g) |  |  |  |  |
|  | Management of dispatch reliability including delays and cancellations |  |  |  |  |  |
|  | Management of systems and equipment failure rates |  |  |  |  |  |
|  | Management of engine reliability, IFSD rate, and applicable EDTO capability |  |  |  |  |  |
|  | Management of CAT II/III system reliability |  |  |  |  |  |
|  | MANAGEMENT SYSTEM |  |  |  |  |  |
|  | SMS |  |  |  |  |  |
|  | Any safety finding relevant to this aircraft type has been identified with root cause and contributing factors to prevent reoccurrence. | CAMO.A.160 CAMO.A.200(a)(1) CAMO.A.202 CAMO.A.205 ORO.GEN.155 ORO.GEN.160 ORO.GEN.200 |  |  |  |  |
|  | Any safety measure/information mandated by CAAT relevant to this aircraft type has been effectively implemented. |  |  |  |  |  |
|  | The mandatory occurrences relevant to selected aircraft registration are reported to the following entities, within 72 hours after being identified: - CAAT Organisation responsible for the design of the aircraft Organization responsible for engine or propeller type design, when related Individual or organization responsible for the design of the modification or repair, when related |  |  |  |  |  |
|  | The mandatory occurrence report and follow-up report for the selected aircraft registration are made in a form and manner established by CAAT with the required information. |  |  |  |  |  |
|  | For occurrence of the selected aircraft registration, hazard and risk assessment have been carried out and/or investigated and mitigated. |  |  |  |  |  |
|  | Any change to aircraft configuration (major modification/repair) for this aircraft type is managed i.a.w. Management of Change procedures. |  |  |  |  |  |
|  | Any aviation safety hazards associated with any contracting or subcontracting of this aircraft type are considered as part of the organisation’s management system. |  |  |  |  |  |
|  | COMPLIANCE MONITORING |  |  |  |  |  |
|  | Sample product audit that is performed by quality function for this aircraft type. | CAMO.A.200(a)(2) |  |  |  |  |
|  | Any quality audit finding relevant to this aircraft type has been addressed and implemented with corrective actions. |  |  |  |  |  |
|  | The following personnel competency for this aircraft type are controlled, trained, and their competency is assessed for the task they performed: - Nominated personnel CAM personnel - Engineering and planning CAM personnel - Reliability and engine monitoring Quality audit personnel – Auditor Airworthiness Review Staff (ARS) Airworthiness Review Staff Examiner (ARSX) | CAMO.A.200(a)(1)(iv) CAMO.A.305 CAMO.A.310 |  |  |  |  |
|  | The subcontracted organisation who performs any subcontracted tasks for this aircraft type is covered and works under the operator’s management system: - Covered by the operator’s training program and competency control - Covered by operator’s audit program - Covered by operator’s SMS - Any changes affecting the contract are notified to the operator by the subcontracted organisation, then to CAAT by the operator | CAMO.A.205 M(L).A.201 |  |  |  |  |
|  | The contract and subcontract organisations performing tasks for this aircraft type are valid and controlled. |  |  |  |  |  |
|  | AIRWORTHINESS REVIEW |  |  |  |  |  |
|  | Sample intervals between AR accomplishment. | CAMO.A.320 |  |  |  |  |
|  | AR is carried out with full documented review of aircraft records and aircraft physical survey. |  |  |  |  |  |
|  | The AR of the aircraft is performed by the authorized CMR staff. |  |  |  |  |  |
|  | The AR report is issued only when the aircraft is airworthy. |  |  |  |  |  |
|  | The AR task is not contracted or subcontracted. |  |  |  |  |  |
|  | The AR report is submitted to CAAT within 10 days. |  |  |  |  |  |
|  | AIRCRAFT PHYSICAL INSPECTION |  |  |  |  |  |
|  | Aircraft exterior inspection, including but not limited to: - Nose Fuselage Wing Landing gear and wheel (and/or floats) Engine including propeller if installed Empennage / Tail APU | CAMO.A.315 |  |  |  |  |
|  | Cockpit inspection, including but not limited to: - Required on-board documents - Operational documents and instructions to flight crews - Cockpit doors and seats - Cockpit safety equipment - Cockpit displays and systems - Cockpit general condition | CAMO.A.315 ORO.MLR.110 SEA.MLR.110 SPO.GEN.140 NCC.GEN.140 NCO.GEN.135 |  |  |  |  |
|  | Cabin/cargo inspection, including but not limited to: - Cabin interior configuration Crew areas, galleys, rest area Toilets / lavatory (if installed) Passenger cabin Cargo compartment | CAMO.A.315 |  |  |  |  |
|  | Operational-Derived Equipment (ODE) or system required for: - Instruments and equipment requirements All granted specific approvals / specialised operations / other authorisations | CAMO.A.315 |  |  |  |  |
|  | OVERALL |  |  |  |  |  |
|  | Access to facility, aircraft, document, records, data, procedures or any other material relevant to its activity subject to certification for this aircraft type, whether it is contracted/ subcontracted or not, are provided to CAAT for determining compliance during this audit. | CAMO.A.140 |  |  |  |  |
|  | The aircraft is maintained in accordance with CAAT Requirements in an airworthy condition under its applicable approval (AOC, OpSpec, Operating license, SFP, or other authorizations). | CAMO.A.315 |  |  |  |  |
|  | The organisation performs activities on this aircraft type in accordance with their approved Terms of Approval, Privileges, and Scope of Work. | CAMO.A.125 |  |  |  |  |

S = Satisfy, U= Unsatisfied, N/A= Not Applicable

Comment= Description the detail of compliance, Non-Compliance details, Reason for not-applicable, or other information

REMARKS:

|  | Assessed by: ………………………………………………… Inspector Name ( ……………………………………………….. ) Position ………………………………………….. Date …………………………………………. |
| --- | --- |